| ZONING DISTRICT | Minimum Lot Dimensions A,S |  | Maximum Building Height T |  |  | Minimum Setbacks B, H, I |  |  | Maximum Lot Coverage J,S | Minimum Per Unit Living Area (Residential Districts) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Lot Area N,O,P Density Q | Lot Width | Stories |  | Feet | Front D | Sides E, L | Rear |  |  |
| AG AGRICULTURAL |  |  |  |  |  |  |  |  |  | 1 story - 1,200 sq. ft. 1 ½ Story - 1st - 1,000 sq. ft. 1 ½ Story - 2nd - 500 sq. ft. 2 Story - 1st - 1,000 sq. ft. 2 Story - 2nd - 1,000 sq. ft. |
| AG-SMALL FARM | 5 Acres O,P,Q | 330’ | 2 ½ C | 35’ C |  | 50’ | 20’k | 50’ | 25% |  |
| AG-RURAL HOMESITE | 2 Acres O,P,Q | 165’ | 2 ½ C | 35’ C |  | 50’ | 20’ k | 50’ | 25% |  |
| RL RESIDENTIAL LOW DENSITY |  |  |  |  |  |  |  |  |  |  |
| RL-CONVENTIONAL -NO SEWER | 1 Acres O,P,Q,R | 150’ | 2 1/2 | 35’ |  | 40’ | 20’ k | 50’ | 30% |  |
| RL-CONVENTIONAL -WITH SEWER | 12,000 P,Q,R Square Feet | 100’ | 2 1/2 | 35’ |  | 30’ | 15’ k | 35’ | 30% |  |
| RM RESIDENTIAL MEDIUM DENSITY- SEWER SERVICE AREA |  |  |  |  |  |  |  |  |  | 1 Story - 1,000 sq. ft. 1 1/2 Story - 1st - 800 Sq. ft. 1 1/2 Story - 2nd - 350 sq. ft. 2 Story - 1st - 750 sq. ft. 2 Story - 2nd - 750 sq. ft. |
| RM, SINGLE FAMILY- NO SEWER | 1 Acre O,P,Q,R | 150’ | 2 1/2 | 35’ |  | 40’ | 20’ k | 50’ | 30% |  |
| RM, SINGLE FAMILY- WITH SEWER | 8,400 P,Q,R Square Feet | 70’ | 2 1/2 | 35’ |  | 30’ | 10’ least 25’ total* k | 30’ | 40% |  |
| RH RESIDENTIAL HIGH DENSITY-SEWER SERVICE AREA |  |  |  |  |  |  |  |  |  | Attached Efficiency = 400 sq. ft. 1 bedroom = 700 sq. ft. 2 bedroom = 850 sq. ft. 3 bedroom = 1,000 sq. ft. 4 bedroom = 1,200 sq. ft. Ground floor = 500 sq. ft / unit Individual condominium units shall contain a minimum of 864 square feet. Maximum building length = 180 feet Detached 1 Story - 864 sq. ft. 1 1/2 Story - 1st - 700 Sq. ft. 1 1/2 Story - 2nd - 300 sq. ft. 2 Story - 1st - 650 sq. ft. 2 Story - 2nd - 650 sq. ft. |
| MULTI FAMILY APARTMENTS, CONDOMINIUMS & ASSISTED LIVING FACILITIES | 15-25 Units per acre G | 200’ | 3U | 40’U |  | 50’I | 40’ I,J ,U | 50’I | 35% building footprint 50% max impervious surface J |  |
| MULTI FAMILY APARTMENTS, CONDOMINIUMS & ASSISTED LIVING FACILITIES | 15-25 Units per acre G | 200’ | 3U | 40’U |  | 50’I | 40’ I,J,U | 50’I | 35% building footprint 50% max impervious surface J |  |
| ATTACHED 4 PLEX | 10 Units per acre G | 100’ | 2 1/2 | 35’ |  | 35’ | 30’ I | 35’ | 25% |  |
| ATTACHED DUPLEX | 8-15 Units per acre G | 100’ | 2 1/2 | 35’ |  | 35’ | 25’ I | 35’ | 25% |  |
| DETACHED SINGLE FAMILY | 6,600 P,Q,R Square Feet | 50’ | 2 1/2 | 35’ |  | 25’ | 9’ least 19’ total | 25’ | 25% |  |

SECTION 1701. HEIGHT, BULK, DENSITY, AREA AND SETBACK REQUIREMENTS BY DISTRICT